PRODUCT  SPECIFICATION

1.PRODUCT  CODE  :  DMC18702

2. DESCRIPTION  :       18 core  7/.2  pcw  v75  insulated  Aluminium / Polyester  tape screened

including  a  7/.2 tinned  drain  wire  V75c  PVC  sheathed.

AUSTEL  A93/CA/0123  Low  voltage  use  ,not  for  mains  connection.

3. CONSTRUCTION

3.1  CONDUCTOR :    7  strands  of  0.2mm  nom.  Annealed  Copper  to

AS1125  drawn  from  Class  110A  copper  to

AS1574.

Max.  D.C. resistance  at  20C  : 89.0 ohms / klm

3.2  INSULATION :    Coloured  P.V.C  to  AS3808  type  V75.

Nominal  Diameter  :   1.15    mm

Nom.  Wall  Thickness:  0.26  mm

3.3 LAY UP  :            18  cores  twisted.

1.Red  2.Blue  3.Green  4.Yellow  5.White  6. Black .

7.  Brown  8. Violet 9. Orange  10. Pink  11. Tan  12. Grey

13. Red/Blue  14. Red/Green 15. Yellow/Red  16. White/Red  17Red/Black   18. Red/Tan

3.4 SCREEN  :           Aluminium / Polyester  tape  screen  in  contact  along

its  length  with  a  7/.2  TACW  drain  wire.

3.5  SHEATH  :         Coloured  P.V.C.  to  AS3808  type  4V75

.

Nominal  Diameter :  7.6    mm

Nom.  Wall  Thickness :  0.76  mm

3.6  IDENTIFICATION :

Legibly  and  Durably  printed  at  approx..  1M  intervals :

“TYCAB AUSTRALIA MULTICORE DATA   DMC8702    CCL  A93/CA/0123”