PRODUCT  SPECIFICATION

1.PRODUCT  CODE    MWS25603

2. DESCRIPTION         	2 Core  Flat  Twin  Sheathed  56/.3  ( 4.0 mm sq) .Tinned copper.                              			V90  insulated  &  Sheathed. MARINE  CABLE

Not  for  Mains GPO Connection.

3. CONSTRUCTION

3.1  CONDUCTOR :  56  strands  of  0.3 mm  Tinned Annealed  Copper  to

AS1125  drawn  from  Class  110A  copper  to

AS1574.

Max.  D.C. resistance  at  20C  :  5.1 ohms / km

3.2  INSULATION :    Coloured  V90  P.V.C. to  AS3808.

Nominal  Diameter  :  3.8   mm

Nom.  Wall  Thickness:  0.6   mm

3.3  CORE  COLOURS

1. Red     2. Black

Laid  parallel.

3.4  SHEATH         :    Coloured  5V90  to  AS3808

Flat  configuration

Nominal  Dimensions  : 5.60  x  9.40  mm

Nom.  Wall  Thickness  : 0.8  mm