PRODUCT  SPECIFICATION

1.PRODUCT  CODE    TMP13703

2. DESCRIPTION          13  Core 7/.3  (0.5sqmm) .P.C.W.  UV stabilised polypropylene insulated.

Score and snap UV stabilised  TYFLO MP .  sheathed . Irrigation  cable.

Extra Low Voltage . Not for Mains  connection.

3. CONSTRUCTION

3.1  CONDUCTOR :  7  strands  of  0.3mm  Annealed  Copper  to

AS1125  drawn  from  Class  102  copper  to

AS1574.

Max.  D.C. resistance  at  20C  : 38.4 ohms / km

3.2  INSULATION :    Coloured  UV stabilised polypropylene

Nominal  Diameter  :  1.6  mm

Nom.  Wall  Thickness:   0.3  mm

3.3  LAY  UP                13   Cores  twisted .

1.  Black   2  Red   3. White  4.Blue  5. Green

6. Brown  7. Yellow  8. Pink  9. Grey  10. Violet

11 . Beige  12. Light Green  13. Orange

3.4  SHEATH               Score and snap UV stabilised  TYFLO MP.

Nominal  Diameter :  8.5  mm

Nom.  Wall  Thickness :    0.7  mm